| SIFDD Specification |
| --- |
| TYPICAL SPECIFICATIONS |  |
| Model: | SIFDD |
| Description: | Fan shall be of the "Square in-line" type with a square inlet and outlet. Fan shall be designed for floor mount or hung installations in-line with ducting, and have a direct driven centrifugal or mixed flow wheel. Fan shall be a corrosion resistant steel, direct driven, square inline blower. |
| Application: | Square In-Line Duct Blowers are suitable for various applications. Mounted indoors with both inlet and outlet ducts to supply or exhaust air. Square In-Line Duct Blowers are engineered to discharge air, fumes and other contaminants horizontally or vertically (UL705 only) in a duct. |
| Certifications: | Square In-line Duct Blower(s) shall be licensed to bear the AMCA Certified Ratings Seal for Air and Sound Performance (210-07 & 300-08) and be ETL Listed to UL705 and/or UL762 Standards. |
| Construction: | Square In-line Duct Blower(s) shall be constructed of galvanized and/or stainless steel. A removable cover shall provide blower motor access, and drive access shall be provided on both sides through removable panels. Blowers shall be direct driven; centrifugal or mixed flow backward inclined. |

Housing

The blower shell shall be constructed of heavy-gauge galvanized steel or stainless steel for UL762 listed fans. Side panels shall be removable for service access.

Wheel

Backward inclined non-overloading, all-aluminum wheel. Balanced in accordance with AMCA standard 204-96, Balance Quality and Vibration Levels for Fans.

Motor & Motor Compartment

Motor shall have permanently lubricated ball bearings and be of voltage, phase, and enclosure as specified. A motor cover shall be supplied as standard.

Product: Fan shall be model SIFDD as manufactured by CaptiveAire Systems.